.\" /*

.\"  * Revision Control Information

.\"  *

.\"  * $Source: /vol/opua/opua2/sis/sis-1.1/common/src/sis/pld/RCS/pld.doc,v $

.\"  * $Author: sis $

.\"  * $Revision: 1.3 $

.\"  * $Date: 1992/05/06 18:58:49 $

.\"  *

.\"

This package manipulates the network for mapping onto the programmable gate arrays.

Two kinds of architectures are targeted: Xilinx-like (table look up) and Actel.

The optimization is targeted towards minimizing 1) the number of blocks, and 2) delay

through the circuit.

XILINX

A node if feasible if it has at most `support' number of inputs. A network

is feasible if all nodes of the network are feasible.

The following structures are used frequently in the routines. To use these routines,

the xln_init_param_t structure must be allocated and all the necessary fields be set

to proper values. The information on the fields to be set for a particular routine is

given after the description of the routine. The fields are as follows:

typedef struct xln_init_param_defn {

int support;                /* fanin-restriction for a node. For xilinx, support = 5 */

int MAX_FANIN;              /* for two-output blocks - see merge_node()*/

int MAX_COMMON_FANIN;       /* for two-output blocks - see merge_node()*/

int MAX_UNION_FANIN;        /* for two-output blocks - see merge_node()*/

int heuristic;              /* overloaded right now - being used for

xln_decomp_for_merging_network() as well as

xln_reduce_levels()

for xln_decomp_for_merging_network() -

ALL_CUBES: consider cubes of all infeasible nodes.

PAIR_NODES: consider cubes of pair of nodes.

(suggested value = ALL_CUBES)

for xln_reduce_levelS() -

1: collapse a node into all fanouts.

2: collapse only into critical fanouts.

(suggested value = 1) */

int common_lower_bound;    /* for xln_decomp_for_merging_network() - do not

consider a cube-pair for extraction if their

number of common inputs is less than

lower_common_bound (suggested value = 2 for Xilinx)*/

int cube_support_lower_bound; /* for xln_decomp_for_merging_network()  - do

not consider a cube if it has

less than cube_support_lower_bound inputs

(suggested value = 4 for Xilinx)*/

int lit_bound;             /* if the infeasible node has greater than lit_bound

literals, do a good decomposition of the node

(i.e. decomp -g) */

int cover_node_limit;      /* apply exact partition_network() if number of nodes in some

subnetwork (or network) is at most this

(suggested value = 30)*/

int flag_decomp_good;      /* could be 0 (do not apply decomp -g), 1 (apply

decomp -g),  2 (pick the better of the previous two) */

int good_or_fast;          /* if = FAST, just apply cube-packing, else (= GOOD)

all decomposition techniques */

int absorb;                /* if 1, use Roth-Karp to move fanins */

int DESPERATE;             /* if Roth-Karp decomposition fails to get a disjoint

decomposition and DESPERATE = 1, call

xl_node_ao_map() */

int RECURSIVE;             /* if 1, recursively apply Roth-Karp decomposition

on the root node */

int MAX_FANINS_K_DECOMP;   /* node considered for Roth-Karp decomposition if has

at most these many fanins (should be at most 32)

(suggested value = 15) */

int COST_LIMIT;            /* nodes with at most this cost to be collapsed in

partial_collapse routine (suggested value = 1) */

XLN_MOVE_STRUCT xln_move_struct;

int collapse_input_limit;  /* consider total collapse of the network if number

of primary inputs is no more than this

(suggested value = 10) */

int traversal_method;      /* 1 then topological traversal, else levels sorted

on the number of nodes at a level (suggested value = 1)*/

} xln_init_param_t;

typedef struct xln_move_struct_defn {

int MOVE_FANINS;       /* if 1, then moving of fanins is invoked. */

int MAX_FANINS;        /* limit on number of fanins of node whose

fanins are to be moved (at most 32)*/

int bound_alphas;     /* for delay: this controls the number of functions

created (set it to 1 for now) */

} XLN_MOVE_STRUCT;

decomposition

--------------

void

karp_decomp_network(network, support, one_node, user_node)

network_t *network;

int support;

int one_node;

node_t *user_node;

Uses Karp_Roth disjoint decomposition to decompose recursively

nodes of the network having fanin greater than

"support". If "one_node" is 1, it just decomposes the

"user_node", else it decomposes all the nodes of the

network whose fanin is greater than "support".

In the present implementation, the

first decomposition found is selected and no attempt

is made to have the best decomposition.

network_t *

xln_exhaustive_k_decomp_node(node, support, MAX_FANINS)

node_t *node;

int support;

int MAX_FANINS;

Call Roth Karp decomp on a node of the network. Try all possible

input partitions for the node if the number of fanins of the node

is no more than MAX_FANINS (if higher,

return NIL). The original node changes as a result of the

decomposition. If it is infeasible, does an xl_ao on the node.

Finds the number of feasible nodes for each partition.

Returns the best subnetwork (corresponding to the decomposition with

minimum number of feasible nodes).

If the node is feasible, or is not an internal node of a network,

NIL is returned.

int

split_network(network, support)

network_t *network;

int support;

Does a decomposition of the network so that all nodes have a fanin

less than the value of support. Uses kernel extraction and AND-OR

decomposition. Returns one on success.

xln_network_reduce_infeasibility_by_moving_fanins(network, support, MAX_FANINS)

network_t *network;

int support, MAX_FANINS;

Given an infeasible network, tries to make nodes of the network

feasible by moving fanins around, i.e., by moving a fanin of an

infeasible node to some other feasible node that can accommodate one

more input. Uses Roth-Karp decomposition.

Fanins of only those nodes are moved which have at most

MAX_FANINS fanins. The resulting network may not be feasible.

xln_network_ao_map(network, support)

network_t *network;

int support;

Given an infeasible network, apply cube-packing heurisitc to do an

AND-OR decomposition. Generates a feasible network.

xln_node_ao_map(node, support)

node_t *node;

int support;

For node belonging to a network, first maps cubes with number of literals

greater than support to a CLB and after that does a best fit decreasing

packing of the cubes. Replaces the node in the network by this set of new

nodes. If node has less than `support' inputs, nothing is done.

xln_improve_network(network, init_param)

network_t *network;

xln_init_param_t *init_param;

Given an infeasible network, generates a feasible network. Depending

on the options in the init_param, tries different decomposition

schemes: like Roth-Karp, cofactoring, xl_ao_map, split etc.

Must set the following fields of init_param: support,

cover_node_limit, lit_bound, flag_decomp_good, good_or_fast, absorb,

DESPERATE, RECURSIVE, MAX_FANINS_K_DECOMP,

xln_move_struct.MOVE_FANINS, xln_move_struct.MAX_FANINS.

xln_improve_node(node, init_param)

node_t *node;

xln_init_param_t *init_param;

Given an infeasible node of a network, replaces it by a feasible set

of nodes. Depending on the options in the init_param, tries different

decomposition schemes.

Must set the following fields of init_param: support,

cover_node_limit, lit_bound, flag_decomp_good, good_or_fast, absorb,

DESPERATE, RECURSIVE, MAX_FANINS_K_DECOMP,

xln_move_struct.MOVE_FANINS, xln_move_struct.MAX_FANINS.

xln_partial_collapse(network, init_param)

network_t *network;

xln_init_param_t *init_param;

Given an infeasible network, first maps each node

of the network. Then collapses each node into its fanouts, remaps the

fanouts. If the number of blocks decreases, accepts the collapse.

Repeats the process until no more gain. Finally, generates a feasible network.

Must set the following fields of init_param: support,

COST_LIMIT, cover_node_limit, lit_bound, flag_decomp_good, good_or_fast, absorb,

DESPERATE, RECURSIVE, MAX_FANINS_K_DECOMP,

xln_move_struct.MOVE_FANINS, xln_move_struct.MAX_FANINS.

xln_cofactor_decomp_network(network, support, mode)

network_t *network;

int support;

float mode;

Cofactors each infeasible node of the network. The mode is

either AREA or DELAY - it determines the cofactoring variable

selecetion.  Assumes initially that  a delay trace using

DELAY_MODEL_UNIT has been done (if mode is DELAY).

If support is 2, this operation will not return a feasible

network. (to be fixed in future).

xln_cofactor_decomp_node(node, support, mode)

node_t *node;

int support;

float mode;

Given a node belonging to a network, cofactors it recursively, till it becomes

feasible (determined by support). Selection of the cofactoring variable

is based on mode. If mode = AREA, then a variable

that occurs in all the cubes in the same phase (pos. or neg) is

selected first. If mode = DELAY, the most critical fanin is selected first.

For DELAY mode, assumes that a delay-trace has been done on the

network. Replaces the node in the network by the cofactored tree.

If support is 2,   this operation will not return a feasible set of nodes.

(to be fixed in future).

xln_decomp_for_merging_network(network, init_param)

network_t *network;

xln_init_param_t *init_param;

Given an infeasible network, decomposes nodes keeping in mind that a

Xilinx CLB can realize two mergeable functions as well. The resulting

network may not be feasible and appropriate decomposition routines

may be needed later.

Must set the following fields of init_param: support,

MAX_FANIN, MAX_COMMON_FANIN, MAX_UNION_FANIN, heuristic,

common_lower_bound, cube_support_lower_bound. See merge_node()

for details on MAX_FANIN, MAX_COMMON_FANIN, MAX_UNION_FANIN.

block count minimization

-------------------------

int

xln_do_trivial_collapse_network(network, support, MOVE_FANINS, MAX_FANINS)

network_t *network;

int support;

int MOVE_FANINS;

int MAX_FANINS;

Traverses the nodes of the network topologically.

First tries to reduce the number of fanins of the node by moving

some of fanins of the node to some other node.

This is done only if MOVE_FANINS flag is 1, and the

number of fanins of the node is at most MAX_FANINS.

Then collapses the node into all its fanouts if none of the fanouts

become infeasible (as determined by support). If it can be collapsed,

it is deleted from the network. Repeats the process until no more

nodes can be collapsed. Can be applied on a potentially infeasible network.

Returns the number of collapsed nodes.

int

xln_do_trivial_collapse_node_without_moving(node, support)

node_t *node;

int support;

Returns 1 if the node can be collapsed into all the fanouts,

without making them infeasible. Does not delete the node from

the network. Otherwise (i.e., if either the node is

not INTERNAL, or some fanout is primary output, or if some fanout

becomes infeasible) returns 0. Then the network remains the same.

Can be applied on an infeasible node.

int

imp_part_network(network, support, MOVE_FANINS, MAX_FANINS)

network_t *network;

int support;

int MOVE_FANINS;

int MAX_FANINS;

First calls xln_do_trivial_collapse_network(

network, support, MOVE_FANINS, MAX_FANINS).

Then finds out all

the (node, fanout) pairs such that node can be collapsed into fanout

without making the fanout infeasible. Associates a cost with each such

pair. This cost reflects the number of new edges created by the

collapse. Then at each step, picks the pair with minimum cost.

Repeats the whole process until no more collapsing can be done. Can be

applied on an infeasible network also.

void

partition_network(network, support, bincov_heuristics)

network_t       *network;

int             support;

int             bincov_heuristics;

The `network' should have all intermediate

nodes with fanin less than or equal to `support'.

Uses Mathony's binate covering algorithm to

minimize the number of nodes in the network.

Uses different heuristics to solve the covering

problem- `bincov_heuristics' ( 0, 1, 2 or 3) is

a flag that specifies which heuristic should be

employed, as described below:

-  0 (exact),

-  1 (Luciano's heuristic),

-  2 (For large examples),

-  3 (automatically switches between 0 and 2)

void

merge_node(network, MAX_FANIN, MAX_COMMON_FANIN, MAX_UNION_FANIN)

network_t *network;

int MAX_FANIN, MAX_COMMON_FANIN, MAX_UNION_FANIN;

Exploits the fact that a Xilinx CLB can realize two functions also,

provided the two functions satisfy the constraints as listed below:

Each of the two functions should not have fanin more than MAX_FANIN,

they should not have more than MAX_COMMON_FANIN many common fanins,

and the union of their fanins should not exceed MAX_UNION_FANIN.

For xilinx XC3090 the numbers are MAX_FANIN = 4, MAX_COMMON_FANIN = 4

and MAX_UNION_FANIN  = 5.

Although the problem is a maximum cardinality matching problem

(solvable in polynomial time), it is formulated as an integer program,

and solved using package Lindo. If Lindo package is not found in the

path, the program exits. If found, the routine lists the pairs of

nodes that were merged. Does not change the network.

network_t *

xln_check_network_for_collapsing_area(network, init_param, roth_karp_flag)

network_t *network;

xln_init_param_t *init_param;

int roth_karp_flag;

Collapses a feasible network if the number of primary inputs of the network

is at most init_param->collapse_input_limit. Applies Roth-Karp

decomposition and cofactoring to the collapsed network. If the number

of nodes of either decomposition is less than the original network,

accepts the better decomposition and returns the new network.

Else, returns NIL. Roth-Karp decomposition is applied only if

roth_karp_flag is 1.

Must specify support, collapse_input_limit. If support = 2, nothing is

done.

delay optimization

-------------------

xln_reduce_levels(network, init_param)

network_t *network;

xln_init_param_t *init_param;

Given a feasible network (preferably in terms of 2-input gates -

obtained say by speed-up), reduces the number of levels, maintaining

the feasibility of the network.

Must set the following fields of init_param: support,

heuristic, collapse_input_limit, traversal_method,

xln_move_struct.MOVE_FANINS, xln_move_struct.MAX_FANINS,

xln_move_struct.bound_alphas.

network_t *

xln_check_network_for_collapsing_delay(network, init_param)

network_t *network;

xln_init_param_t *init_param;

Collapses a feasible network if the number of primary inputs of the network

is at most init_param->collapse_input_limit. Applies Roth-Karp

decomposition and cofactoring techniques on the network and evaluates

them. Returns the network with lower number of levels. Returns NIL if collapsing

could not be done. Does nothing if init_param->support = 2.

Must set the following fields of init_param: support, collapse_input_limit.

`network' changes if collapsing is done.

ACTEL

Mapping onto ACT1 architecture: minimizing the number of blocks used.

/* init_param structure to put all the options for actel mapping */

typedef struct act_init_param_defn{

int HEURISTIC_NUM;  /* what BDD (ordered or unordered) to construct */

int NUM_ITER;       /* number of iterations in iter. improvement */

int FANIN_COLLAPSE; /* upper limit on fanins of a node to collapse */

float GAIN_FACTOR;  /* go from one iteration to other only if gain increases

by this ratio (suggested value = 0.001) */

int DECOMP_FANIN;   /* lower limit on fanins of node to decompose */

int DISJOINT_DECOMP;/* do disjoint decomposition before mapping */

int QUICK_PHASE;    /* do phase-assignment */

int LAST_GASP;      /* make network out of node and do iterations on it */

int BREAK     ;     /* make the final network in terms of basic blocks */

float mode;         /* 0 for area and 1 for delay mode, in between, a

weighted sum */

char delayfile[500];/* name of the file with delay numbers for basic block

as a function of number of fanouts - needed only if

mode is non-zero */

} act_init_param_t;

network_t *

act_map_network(network, init_param, cost_table)

network_t *network;

act_init_param_t *init_param;

st_table *cost_table;

This procedure synthesizes the given circuit onto actel

architecture. The original network "network" is lost.

It uses tree-mapping approach to

cover the subject graph with the pattern-graphs.

The pattern graphs are hardwired into the code

and so no library is to be read in.

Subject graph and pattern-graphs are in terms of

2-1 muxes. Subject graph is constructed for each intermediate

node of the network. Either an OBDD (Ordered reduced BDD)

and/or a BDD (Binary Decision Diagram) is constructed for each

such node. After entire network is mapped, an iterative_improvement

phase may be entered.

The following fields of init_param must be set: HEURISTIC_NUM,

NUM_ITER, FANIN_COLLAPSE, DECOMP_FANIN, DISJOINT_DECOMP,

QUICK_PHASE, GAIN_FACTOR, LAST_GASP, BREAK, mode.

HEURISTIC_NUM (1, 2, 3, 4) specifies which one of the two

subject_graphs would be constructed:

HEURISTIC_NUM = 1 => OBDD

HEURISTIC_NUM = 2 => BDD

HEURISTIC_NUM = 3 => program decides which one to construct.(default).

HEURISTIC_NUM = 4 => both are constructed and the one with

lower mapped cost is selected. Gives the best

result, but typically takes more time.

NUM_ITER equals the maximum number of iterations

to be performed in the iterative_improvement phase. Each such

iteration involves a good_decomposition followed by a partial_collapse

routine.

If the iterative_improvement phase is entered, in the partial_collapse

routine, only those nodes are considered as candidates

for partial_collapse which have fanin no more than FANIN_COLLAPSE.

The good_decomposition routine is called only for those nodes

which have fanin greater than or equal to DECOMP_FANIN.

The program enters the next iteration only if gain in the present

iteration is at least (present_cost * GAIN_FACTOR).

The cost and gain are measured in terms of the number of basic blocks

used for the network.

If DISJOINT_DECOMP is non-zero, disjoint_decomposition routine

is invoked on the network at the very start. This may help in

getting big gains in partial_collapse routine later. Also, big

nodes are decomposed, potentially reducing the final cost.

If QUICK_PHASE is non-zero,  the program, before exiting,

enters an act_quick_phase routine, which greedily

finds out if it is beneficial to implement the node in

negative phase.

if BREAK = 1, each node in the final network will be implementable by

one ACT1 block. A bdnet file is also generated. There is a global

variable FILE *BDNET_FILE which must be fopen-ed by the user

before calling the routine with BREAK = 1. Then it is his

responsibility to fclose it.

mode must be set to AREA for the time being.

mode = DELAY is not yet supported.

A global variable `act_is_or_used' should be set before calling this routine.

It should be set to 1 if the OR gate in ACT1 is to be used, else set to 0.

The cost_table should be initialized by

st_init_table() before calling this routine. After the routine, it has

a subject graph for each node. Call free_cost_table() to free all the

associated storage. Also it is a serious error to call act_map_network()

with negative flags.

void

free_cost_table(cost_table)

st_table *cost_table;

Frees the associated storage associated with the

cost_table. This space was allocated by the act_map_network()

routine.

Some utility routines

---------------------

int

xln_is_network_feasible(network, support)

network_t *network;

int support;

Returns 1 if the network is feasible, as determined by support.

Else returns 0. (For xilinx)

pld_replace_node_by_network(node, network)

node_t *node;

network_t *network;

Replaces node by network. Node should be in some network.

node_t *

pld_make_node_from_cube(node, cube)

node_t *node;

node_cube_t cube;

Returns a node which has the same function as `cube', which belongs to

the `node'.