EECS140 Midterm 2

Spring 2016

| Prob. | Score |
| --- | --- |
| 1 | /10 |
| 2 | /29 |
| 3 | /16 |
| 4 | /10 |
| 5 | /15 |
| Total | /80 |

Name__________________________

SID___________________________

- For the op-amp below, assume that:

- All transistors are biased in saturation

- All capacitors are assumed to be zero except CC, C2 , and Cgs for all transistors.

- gmro >> 1 for all combinations of gm and ro

2B

2A

CC

C2

5

1A

4

1B

3

6

The same op-amp will be used in several different feedback circuits.

  - [2] The spec requires that the output be able to swing to within 200mV of ground, and 150mV of the positive supply.  List all constraints on overdrive voltages.

  - [2] When used in feedback the closed-loop gain must be 5 +/- 1%.  What is the constraint on the open-loop gain of the amplifier?

  - [2] When used in feedback with a closed-loop gain of 5, the desired bandwidth is 200M rad/s.  What is the constraint on the unity-gain bandwidth?

  - The amplifier is also to be used in unity-gain feedback driving a 100pF load, and the desired open-loop phase margin is 70 degrees.

    - [2] What is minimum possible value for gm4? (answer should be in Siemens)

    - [2] What are the constraints on gm1 and Itail? (in terms of other circuit values)

- A particular design of the op-amp above has the following values for parameters.  You may ignore the gain and phase associated with the current mirror for this problem.

| Gm1 | Ro1 | Gm2 | Ro2 | C2 | CC | Cgs4 |
| --- | --- | --- | --- | --- | --- | --- |
| 1mS | 1M | 1mS | 100k | 100p | 1p | 0.1p |

- [6] What are the low-frequency gains, and the locations of the uncompensated (CC=0) poles, uncompensated second-stage unity gain frequency, and right-half-plane zero associated with Cc?

| ¦Av1,0 ¦ | ¦Av2,0¦ | p1 | p2 | u2 | z.RHP |
| --- | --- | --- | --- | --- | --- |

On the following pages,

- [3] plot the magnitude of the second stage gain

- [6] plot the overall impedance seen at the first stage output,

- [2] plot the magnitude of the first stage gain,

- [6] plot the magnitude and phase of the overall gain.  Label any poles and zeros clearly.

- [4] Estimate the unity-gain phase margin for this value of CC.  What value of feedback factor f gives a 45 degree phase margin?

- [2] Approximately what value of CC is needed for a 45 degree phase margin if we can ignore z,RHP?

1

1k

1M

1G

1

100

10,000

|Av2|

|Av2|

1

1k

1M

1G

1

1k

1M

1G

1K

1M

|Zo1|

|Zo1|

2C) Impedance at first stage output,  |Zo1|

2B) Second stage gain – |Av2,0|

2D) First stage gain,  |Av1|

rad/s

rad/s

rad/s

1

1,000

|Av1|

|Av1|

1

1k

1M

1G

-180

-90

0

Phase(Av)

1

1k

1M

1G

Phase(Av)

-270

1

100

10,000

|Av|

|Av|

2E) op amp Bode plot

rad/s

rad/s

Label any poles and zeros clearly!

Don’t forget RHP zero

Don’t forget to answer 2f and 2g (phase margin questions)

- [16] For the four differential amplifiers below, assume that 1/gm << R << ro for all configurations, and that current sources are ideal.  Calculate the transconductance Gm and the output resistance Ro for each amplifier.  Assume a purely differential input.

2B

2A

1A

1B

VoutD

1A

1B

1A

1B

VoutC

1A

1B

Rtail

RD

VoutB

VoutA

Amp A

Amp B

Amp C

Amp D

VB2

RD

RD

Rtail

|  | Gm | Ro |
| --- | --- | --- |
| Amp A |  |  |
| Amp B |  |  |
| Amp C |  |  |
| Amp D |  |  |

- [10] The audio amplifier circuit below consists of a two-stage op-amp with an output stage.  The op-amp is wired up in feedback.

  - Which transistors make up the input differential pair?

  - Estimate the tail current in terms of circuit parameters.

  - Which transistors make up the second stage?

  - Which capacitor provides Miller compensation?

  - What component minimizes the effect of the right-half-plane zero associated with Miller compensation?

  - What is the DC bias voltage on the base of Q1?

  - What is the low-frequency feedback factor, fDC? (you may ignore finite BJT input impedance)

  - What is the mid-frequency feedback factor, faudio?

  - What RC time constant determines the transition from low-frequency to mid-frequency?

(source: eeweb.com/blog/circuit_projects)

- For the amplifier in the figure to the right

RD

RS

Vo

Vi

[4] Draw the small signal model labeling the small signal variables vi, vo, io, vd

  - [1] Write an expression for Gm as the ratio of two small signal parameters while a third is held equal to zero.

  - [1] Write vd in terms of io

  - [4] Write KCL @ vo and solve for Gm.

  - [3] Find the approximate value for Gm for each of three different values of RD: much less than ro, equal to ro, and much greater than ro.

  - [2] Write the full expression for Ro. (you don’t need to derive it)