EE 147/247A	Prof. Pister

Fall 2015

Homework Assignment #10

Due by online submission Wednesday 11/11/2015 (Thursday at 9am)

- Check out the SOI-MUMPS web page http://www.memscap.com/products/mumps/soimumps

  - How often do they run the process?  How long does it take them from the time you submit your design (the design deadline) until they ship you the parts?

  - What is the minimum cost for 15 dice for academics?

  - If you were a small company making prototype products and needed hundreds of dice, what would the cost per square millimeter be?  How does this compare to the cost of CMOS?

- Download the SOI-MUMPS design rules (available on the reference page).  Read section 2.2, 2.3, and 2.4 on design rules

  - What is the minimum size of an SOI square with a square of PADMETAL on it?

  - What is the minimum distance from an SOI feature to a TRENCH feature to guarantee that the edge of SOI will be not be over the trench?

  - What is the maximum width of an SOI beam (that is NOT over TRENCH) that is guaranteed to be released?

  - How close do etch holes need to be in an SOI plate (that is not over TRENCH) in order for it to be released?

  - What is the maximum fraction of the SOI layer that you are allowed to etch in your design?

  - Why do they recommend that you put a fillet on beams?

- Pick one of the following devices and lay it out for fabrication in the class SOI run. Note that this is a single mask process so you will need to use etch holes to release any large structures. Design rules and wafer properties are given below. Export your mask file as a GDSII with the filename format <your_lastname>.gds and upload it along with your homework.

  - Comb-drive resonator

  - Capacitive accelerometer

  - Thermal actuator (“heatuator”)

  - Electrostatic relay

  - Electrostatic logic element (NOT, NAND, etc.)

  - Pull-in accelerometer (uses the time necessary to close a gap (like a relay) to measure acceleration

  - Robot leg

  - Silicon fracture test structure

  - anything else you come up with as long as you get approval from ksjp

- For each of references [1]—[4] which use standard CMOS as the baseline process followed by relatively simple post-processing

  - What is/are the structural material(s)?

  - What is/are the sacrificial material(s)?

  - What is/are the sacrificial etchant(s)?

- [247] If you could assemble chips from SOIMUMPS, POLYMUMPS, and standard CMOS, could you make a better robot than the one in Hollar et al. did [5]?

[1] G. Fedder, “Integrated MEMS in Conventional CMOS”, Tribology Issues and Opportunities in MEMS, 1997.  (Lots of things covered in this paper – focus on figure 2).

http://www.eecs.berkeley.edu/~pister/147fa15/Resources/Fedder97.pdf

[2] C.Wang, M. Tsai, C. Sun, W. Fang, “A novel CMOS out-of-plane accelerometer with fully differential gap-closing capacitance sensing electrodes”, J. Micromechanics and Microengineering, V17, 2007.

http://www.eecs.berkeley.edu/~pister/147fa15/Resources/Wang2007.pdf

[3] C. Lin, C. Hsu, C. Dai, Fabrication of a Micromachined Capacitive Switch Using the CMOS-MEMS Technology, Micromachines, Nov 2015.

http://www.eecs.berkeley.edu/~pister/147fa15/Resources/Lin2015.pdf

[4] A. Jain, H. Xie, “A single-crystal silicon micromirror for large bi-directional 2D scanning applications”, Sensors and Actuators, A, 2006.

http://www.eecs.berkeley.edu/~pister/147fa15/Resources/Xie2006.pdf

[5] S. Hollar, A. Flynn, S. Bergbreiter, K. Pister, “Solar Powered 10 mg Silicon Robot”, MEMS, 2003.

http://robotics.eecs.berkeley.edu/~pister/publications/2003/hollar_mems03%20solar%20powered%20robot.pdf

| Design Rules | Dimension |
| --- | --- |
| Min. Feature Size (line and space) | 2um |
| Target Oxide Undercut | 10um |
| Min. Etch Hole Size | 4x4um2 |
| Min. Anchor/Pad Size | 50x50um2 |

| Wafer Properties |  |
| --- | --- |
| Device Layer Thickness | 15um |
| Sheet Resistance | 4 ohms/square |
| Oxide Thickness | 2um |
| Handle Wafer Thickness | 500um |