EPIC: A new generation of architectures  beyond RISC

Presentation at UC Berkeley

by

Mike Schlansker
Hewlett-Packard Laboratories
March 3, 1999

The uniprocessor design problem:

	How to provide ever increasing performance for existing applications without re-writing them?

	Modest speedups (e.g.  2) are critical!

	How to exploit expanding VLSI circuit area?

	Substantial area should be devoted to Instruction-Level Parallelism (ILP)

	How to issue many operations per cycle?  (4-16)

	How to tolerate long pipelined latency?  (1-50)

	How to cope with a large number of operations in flight?  (10’s to 100’s)

EPIC: a paradigm shift

	RISC solution

	Based on sequential execution semantics

	Compiler’s role is limited by the instruction set architecture

	Superscalar hardware identifies and exploits parallelism

	EPIC solution – (the evolution of VLIW)

	Based on parallel execution semantics

	EPIC ISA enhancements support static parallelization

	Compiler takes greater responsibility for exploiting parallelism

	Compiler / hardware collaboration often resembles superscalar

Why pursue new architectures?

	Disadvantages of continuing a pure out-of-order superscalar approach

	Out-of-order execution is complex for high ILP

- expensive in real estate

- difficult to debug & verify

	No existence proof at high levels of ILP

	Advantages of pursuing EPIC architectures

	Make wide issue & deep latency less expensive in hardware

	Allow processor parallelism to scale with additional VLSI density

EPIC philosophy

	Architect the processor to do well with in-order execution

	Enhance the ISA to allow static parallelization

	Use compiler technology to parallelize program

	However, a purely static VLIW is not appropriate for general-purpose use

The fusion of VLIW and superscalar techniques

	Superscalars need improved support for static parallelization

	Static scheduling

	Limited support for predicated execution

	VLIWs need improved support for dynamic parallelization

	Caches introduce dynamically changing memory latency

	Compatibility: issue width and latency may change with new hardware

	Application requirements - e.g. object oriented programming with dynamic binding

	EPIC processors exhibit features derived from both

	Interlock & out-of-order execution hardware are compatible with EPIC    (but not required!)

	EPIC processors can use dynamic translation to parallelize in software

Many EPIC features are taken from VLIWs

- Minisupercomputer products stimulated VLIW research (FPS, Multiflow, Cydrome)

- 	Minisupercomputers were specialized, costly, and short-lived

- 	Traditional VLIWs not suited to general purpose computing

- 	VLIW resurgence in single chip DSP & media processors

- Minisupercomputers exaggerated forward-looking challenges:

- 	High-speed clock

- 	Long latency

- 	Wide issue

- 	Large number of architected registers

- 	Compile-time scheduling to exploit exotic amounts of parallelism

- EPIC exploits many VLIW techniques

- VLIW compilers must create

- an efficient ILP record of execution

- Trace scheduling (Fisher at Multiflow)

- Use static program statistics to identify traces

- Schedule code across basic blocks to maximize on-trace performance

- Treats arbitrary scalar programs

- Speculative code motion

- Basic blocks are short & sufficient ILP requires motion across blocks

- Speculation reduces height of program schedule by changing operation’s execution condition

- Exceptions must be ignored – dismissable load (Multiflow)

- Modulo scheduling (Cydrome)

- Near optimal performance for innermost-loops

- More flexible than vector - treats recurrences and while loops

- Specialized branch and rotating registers eliminate code replication (Rau)

- Predicated execution of if-converted code

- Efficient treatment of many unbiased conditionals

- Allow code motion without changing operation’s execution condition

- Predicated execution: hardware support

| compare operation | predicated operation |
| --- | --- |

VLIW architectural features prior to 1989

- Wide issue (MultiOp)

- Exposed latency

- Large number of architected registers

- Dismissable loads

- Rotating registers & specialized branches to support software pipelining

- Predicated execution

- Execution of multiple prioritized branches per instruction (Multiflow)

- Shortcomings of first generation of VLIWs

- Expensive multi-chip implementations

- 	No data cache

- Poor "scalar" performance

- No strategy for object code compatibility

- EPIC design challenges

- 	Develop architectures applicable to general-purpose computing

- 	Find substantial parallelism in “difficult to parallelize” scalar programs

- 	Provide compatibility across hardware generations

- 	Support emerging applications (e.g. multimedia)

- 	Compiler must find or create sufficient ILP

- 	Combine the best attributes of VLIW & superscalar RISC                           (incorporated best concepts from all available sources)

- 	Scale architectures for modern single-chip implementation

- HPL PlayDoh: a testbed for EPIC architectures

- 	PlayDoh - HP Laboratories virtual processor for ILP research

- 	Virtual platform provides compiler & simulator for  PlayDoh ISA

- 	Publicly available for external research

- 	Remainder of talk highlights EPIC features found in PlayDoh

- The second generation: some key EPIC features

- 	Generational compatibility

- 	Improved support for control speculation

- 	Programmatic control of the cache hierarchy

- 	Data speculation

- 	Improved support for predication

- 	Unbundled & concurrent branches

- Latency compatibility

- 	Schedule code using actual hardware latencies of target processor

- 	Compile with “less than or equals semantics”  to allow early arrival

- 	Two approaches use hardware for late operand arrival:

- 	Use exposed latency semantics & “latency stalling” (tardy operand stalls)

- 	Use unit latency semantics & interlocks (tardy operand stalls if used)

- Control speculation and exception reporting

- 	Need: accurate error reporting

- 	Need: ability to handle exceptions

- Deferred exception handling

- 	Speculative & non-speculative opcodes, operand tags, propagation rules   [Cydrome; Ebcioglu; Mahlke, et al.]

- 	 Software responsible for error isolation and recovery

- Problems involving data caches

- Absence of a data cache increases critical path length (especially for scalar code)

- Presence of data cache degrades performance with poor data locality (cache thrashing and cache trashing)

- Non-deterministic latencies are a problem for static scheduling

- 	Assuming cache hits results in total dead time when the cache misses

- 	Assuming cache misses results in pessimistic schedules

- EPIC objectives: enhance run-time determinism using compile-time data cache management

- Programmatic data cache management

| bypass & latency specifying loads | pretouch and prefetch |
| --- | --- |

- Compiler directs motion of data through cache hierarchy

[Convex C-1; Intel i860; Chi & Dietz 1989]

Memory dependences and static scheduling

“May” dependences on the critical path

| Original Code *a=*b *c=*d *e=*f |  | Schedule height = 12 Memory latency =10 ? |
| --- | --- | --- |

	Cache alleviates problem by reducing latency - but may cause pollution

Data speculation

|  | Schedule height = 9 |  | Schedule height = 5 |
| --- | --- | --- | --- |
| Dependence graph with 2-cycle cache latency |  | Dependence graph with data speculation |  |

	Data speculation: Stores in lds-ldv window checked in hardware [Chen 93]

	Does not require cache and cache pollution

Improved architecture for predicates

|  | PlayDoh code for C conditional “r=(a<b)?a:b” is written: p1,p2=cmpp.w.<.UN.UC(a, b)if p r = a if p1 r = b if p2 |
| --- | --- |

| Predicate input | Compare result | UN | UC |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

	Compound & two-target compare minimizes latency & op count

	Unconditional actions UN and UC support simple & nested conditionals

Unconditional compares do not support general  if-conversion

|  | A if p p1, p2, p3 = ... B if p1; p4 = cmpp.w.<.UC(a, b) C if p2; p4 = cmpp.w.<.UC(c, d) D if p3 p4 = cmpp.w.>.UN(e, f) E if p4 Wrong code |
| --- | --- |

	For unstructured code:

	A merge block has multiple operations that set its predicate

	Need to compute:  p4 = OR(  p1!(a < b) ,   p2!(c < d) ,   p3(e > f))

	Results are incorrect if unconditional (UN, UC) compares are used

Solution "Wired-or compares"

| ## Allows multiple writes per cycle ## If they write identical values, the result is well-defined ## Ops can be issued in any order or concurrently |  |
| --- | --- |

| Predicate input | Compare result | ON | OC |
| --- | --- | --- | --- |
| 0 | 0 | -- | -- |
| 0 | 1 | -- | -- |
| 1 | 0 | -- | 1 |
| 1 | 1 | 1 | -- |

	OR class compares use target action modifiers ON and OC

Wired or compares (cont)

	An n-term wired or is calculated with:

	a single clear

	n unary or operations

The following sequence computes predicate p4 as defined above:

p4 = 0

p4 = cmpp.w.<.OC(a, b)  if p1

p4 = cmpp.w.<.OC(c, d)  if p2

p4 = cmpp.w.>.ON(e, f)  if p3

Chains of predicate computations inhibit ILP

|  | Linked compare computation If-converted code: ... p2 = cmpp.w.=.UN(t, 'M') if T p3 = cmpp.w.>.UC(r, 0) if p2 p5 = cmpp.w.=.UN(r, 0) if p3 s = 0.8 * s if p5 ... |
| --- | --- |

Need to compute:  p5=AND(  (r=0),   !(r>0),   (t=‘M’)  )

"Wired and" works like "wired or" for fast evaluation of n-way AND

Conventional conditional branch operation

	 Problem: compound branch components are statically co-located

Unbundled conditional branch sequence

[Schorr 1971, Young & Goodman 1984]

Concurrent branching

using Fully-Resolved Predicates (FRPs)

| Branch dependences | Unordered branches with FRPs |
| --- | --- |

Summary: key EPIC  features

	Motivations

	To avoid out-of-order execution

	To enhance ILP on "sequential" computations

	To address the cache miss penalty

	Some EPIC architectural features

	Compatibility strategy for evolving implementation parallelism

	Control speculative opcodes with exception tags & propagation

	Programmatic control of cache hierarchy

	Data speculative loads

	Improved predication (two-target & unordered compares)

	"Unbundled" branch architecture

	Concurrent branching

Compiling for EPIC?

	Complex portfolio of architectural innovations

	New optimizations reduce path length rather than operation count

	New compiler techniques will evolve for EPIC architectures

	Expose ILP - analyze code to maximize available parallelism

	Enhance ILP - transform code to increase available parallelism

	Exploit ILP - map code onto available hardware to maximize efficiency

	Manage the cache hierarchy

	Extend traditional analyses and optimizations for predicated code

	Object code translation

Control CPR - example compiler optimization

| Superblock CFG | Insert fall-through branch & lookahead compares | Branches & compares move off-trace |
| --- | --- | --- |

Summary

	Uniprocessor performance remains a key product objective

	EPIC processors:

	Take advantage of evolving VLSI density

	Exploit additional instruction-level parallelism using enhanced ISA

	Shift responsibility for exploiting parallelism from hardware to software

	Are designed to address general computing needs

	Performance improves as EPIC compilers mine more ILP over a wider range of applications