UNIVERSITY OF CALIFORNIA, BERKELEY

College of Engineering

Department of Electrical Engineering and Computer Sciences

EE 105: Microelectronic Devices and Circuits	Spring 2008

# Final Exam

Time allotted:  3 hours

NAME:  _________________________	_________________________	_________________________

(print)		Last				First				Signature

STUDENT ID#:	____________________

INSTRUCTIONS:

- Use the values of physical constants provided below.

- SHOW YOUR WORK.  (Make your methods clear to the grader!)

- Clearly mark (underline or box) your answers.

- Specify the units on answers whenever appropriate.

| PHYSICAL CONSTANTS. Description; Symbol; Value. Electronic charge; q; 1.610-19 C. Boltzmann’s constant; k; 8.6210-5 eV/K. Thermal voltage at 300K; VT = kT/q; 0.026 V. Note that VT ln(10) = 0.060 V at T=300K. PROPERTIES OF SILICON AT 300K. Description; Symbol; Value. Band gap energy; EG; 1.12 eV. Intrinsic carrier concentration; ni; 1010 cm-3. Dielectric permittivity; Si; 1.010-12 F/cm. | Electron and Hole Mobilities in Silicon at 300K |
| --- | --- |

SCORE:

- 2 extra blank sheets are provided on page 13 and 14 in case you need more space for your answers. Make sure to include a pointer to the overflow space in your answer.

Problem 1 [15 pts] PN Junction Diode

- A Si PN junction consists of equally doped N and P region, ND = NA = 1017 cm-3. Both P and N regions are 1 µm thick, and the junction area is 100x100 µm2.

  - What is the built-in potential of the diode?

  - What is the small signal equivalent circuit of the diode when the forward bias current of the PN junction is 1 mA? Find the values of the circuit elements in the equivalent circuit.

  - What is the small signal equivalent circuit of the diode when it is reverse biased at 5V? Find the values of the circuit elements.

Problem 2 [24 pts] Basic Concepts

- Please answer the following questions.

| Question | Answer |
| --- | --- |
| - [3 pt] (Single choice) The current in the collector region of an NPN bipolar transistor is conducted by (A) electron, (B) hole, (C) both. |  |
| - [3 pt] (Single choice) The drain current in a PMOS FET is carried by (A) electron, (B) hole, (C) both. |  |
| - [3 pt] (Single choice) The channel of an NMOS FET is doped to be (A) N type, (B) P type, (C) neither. |  |
| - [3 pt] (Single choice) Reducing the gate oxide thickness of an MOSFET will (A) increase its transconductance (gm), (B) decrease its transconductance, (C) have no effect. Assume the MOSFET is biased at a fixed drain current. |  |
| - Answer and reason for d) |  |
| - [3 pt] (Single choice) Reducing the thickness of the base layer in a BJT will (A) increase its transconductance (gm), (B) decrease its transconductance, (C) have no effect. Assume the BJT is biased at a fixed collector current. |  |
| - Answer and reason for e) |  |
| - [3 pt] What is the main benefit of replacing gate oxide by high-k dielectric materials in MOSFET? |  |
| - Answer for f): |  |
| - [3 pt] What are the causes of the output resistances for long- and short-channel MOSFET, respectively |  |
| - Answer for g): |  |

Problem 3 [25 pts] BJT Amplifier

- Consider the bipolar amplifier shown below.

|  | - BJT Parameters - IS = 10-16 A - VA = 50V - β = 100 - C = 20 fF - Cμ = 5 fF - CCS = 10 fF - Circuit Parameters: - VCC = 3V - RE = 5 kΩ |
| --- | --- |

  - [2 pt] What is the topology of this amplifier (common emitter, …etc)?

  - [3 pt] Find the expression of the voltage gain, expressed in terms of small-signal parameters.

  - [4 pt] Find the expressions of the input and output resistance.

  - [4 pt] What DC voltage, VDC, is required to bias the transistor so the transconductance of the BJT is 40mS?

  - [4 pt] Find the numeric value of the voltage gain.

  - [4 pt] Draw the relevant capacitances on the circuit diagram above. How many poles does this circuit have?

  - [4 pt] Find the 3-dB bandwidth of this circuit.

Problem 4 [20 pts] MOS Amplifier

- Consider the following amplifier:

|  | NMOS: VTH = 0.5 V, μnCox = 200 μA/V2, λ = 0.1V-1 PMOS: VTH = -0.5 V, μpCox = 100 μA/V2, λ = 0.1V-1 RS = 100Ω |
| --- | --- |

  - [5 pt] What is the function of this amplifier? Explain the functions of each transistor (M1 to M6).

  - [5 pt] Find the expression of the voltage gain.

  - [5 pt] Find the expression of the input and output resistance.

  - [5 pt] Find the numeric value of the output resistance.

Problem 5 [20 pts] Cascode Amplifier

- For the cascode amplifier shown below:

|  | NMOS: VTH = 0.5 V, μnCox = 200 μA/V2, λ = 0.1V-1 PMOS: VTH = -0.5 V, μpCox = 100 μA/V2, λ = 0.1V-1 IREF = 100 µA (W/L)5 = (W/L)6 = (W/L)7 = 10 (W/L)2 = (W/L)3 = (W/L)4 = 10 (W/L)1 = 40 |
| --- | --- |

  - [2 pt] Find the expression of the voltage gain (you can keep only the dominant term).

  - [3 pt] Find the expression of the output resistance (you can keep only the dominant term).

  - [5 pt] Find the voltage at Node X, Y, and A, B.

  - [5 pt] In order for all transistors to stay in saturation, what is the maximum and minimum voltage at output?

  - [5 pt] If the widths of M2, M3, and M4 are doubled (i.e., (W/L)2 = (W/L)3 = (W/L)4 = 20), how do the voltage gain and output resistance change?

Problem 6 [20 pts] Differential Amplifier

- Consider a differential amplifier shown below:

| Q 4 Q 3 + – + – | IEE = 5.2 mA VA = 52 V for NPN transistor VA = 26 V for PNP transistor β = 100 for all transistors |
| --- | --- |

  - [5 pt] Find the expression and numeric value of the voltage gain of the differential amplifier.

  - [5 pt] If the current source has a shunting (i.e., in parallel with IEE) resistance of 100KΩ, and the early voltages of Q3 and Q4 differ by 1%, find the common mode to differential mode voltage gain (both expression and value).

  - [5 pt] What is the common mode rejection ratio (CMRR) of the amplifier in Part b)?

  - [5 pt] The circuit below is also a differential amplifier. How does it differ from the circuit above? Does it still provide immunity to common mode noise?

+ –

(Additional Pages)

(Additional Pages)