UNIVERSITY OF CALIFORNIA, BERKELEY

College of Engineering

Department of Electrical Engineering and Computer Sciences

EE 105: Microelectronic Devices and Circuits	Spring 2008

# MIDTERM EXAMINATION #1

Time allotted:  80 minutes

NAME:  _________________________	_________________________	_________________________

(print)		Last				First				Signature

STUDENT ID#:	____________________

INSTRUCTIONS:

- Use the values of physical constants provided below.

- SHOW YOUR WORK.  (Make your methods clear to the grader!)

- Clearly mark (underline or box) your answers.

- Specify the units on answers whenever appropriate.

| PHYSICAL CONSTANTS. Description; Symbol; Value. Electronic charge; q; 1.610-19 C. Boltzmann’s constant; k; 8.6210-5 eV/K. Thermal voltage at 300K; VT = kT/q; 0.026 V. Note that VT ln(10) = 0.060 V at T=300K. | PROPERTIES OF SILICON AT 300K. Description; Symbol; Value. Band gap energy; EG; 1.12 eV. Intrinsic carrier concentration; ni; 1010 cm-3. Dielectric permittivity; Si; 1.010-12 F/cm. |
| --- | --- |

Electron and Hole Mobilities in Silicon at 300K

SCORE:	1 __________ / 25

2 __________ / 25

3 __________ / 30

Total: 	   __________ / 80

Problem 1 [25 points]: Semiconductor Basics

- A Si resistor is doped with 1017 cm-3 of phosphorus and 2x1017 cm-3 of boron impurities.

i) What are the electron and hole concentrations, n and p, in this sample at room temperature?  [4 pts]

ii) Estimate the resistivity of this sample.  [5 pts]

iii) Qualitatively (no calculations required), how would the resistivity change when the temperature goes up to 100ºC?  Explain briefly.  [4 pts]

- Consider the two Si p-n junction diode below:

i) Find the ratio of the built-in voltages for these two p-n junctions.  [4 pts]

ii) What is the ratio of the current densities under a forward bias voltage of 1V for these two diodes?  [4 pts]

iii) Find the ratio of the areal junction capacitances of these two p-n junctions when they are not biased (i.e., 0V).  [4 pts]

Problem 2 [25 points]: Bipolar Junction Transistor (BJT)

a)     The following two NPN BJTs have the same doping concentrations. The only difference is their base widths: BJT-A has a base width of 100 nm, while BJT-B has a base width of 200 nm. Find the ratio of their current gains. (If you give correct qualitative answer, i.e., which BJT has higher current gain and why, you will get half credit). [6 pts]

b) Consider the following two BJTs. They have identical dimensions and doping profiles, except BJT-A is NPN transistor and BJT-B is PNP transistor.  Find the ratio of their current gains. (If you give correct qualitative answer, i.e., which BJT has higher current gain and why, you will get half credit). [6 pts]

c) Answer this question qualitatively. For the two BJTs in Part a), which BJT will have larger Early voltage? Why? [4 pts]

d) Solve the bias point of the following PNP transistor (IC, VEB, VEC). Assume   [5 pts]

e) Draw the small-signal model of the circuit in Part d). Specify all the small signal parameters used (e.g., , , etc). [4 pts]

Problem 3 [30 points]: BJT Amplifiers

a) Consider the BJT amplifier shown below with. 
Assume.

i) Find the value of . [4 pts]

ii) Is the BJT in the active mode? Why? [4 pts]

iii) Find the small signal parameters of the BJT under this bias condition. [4 pts]

iv) What is the expression for the voltage gain?  What is its numerical value? [6 pts]

v) What is the expression for the input impedance (seen by )? What is its numerical value? [6 pts]

vi) What is the expression for the output impedance (seen by )? What is its numerical value? [6 pts]