UNIVERSITY OF CALIFORNIA, BERKELEY

College of Engineering

Department of Electrical Engineering and Computer Sciences

EE 105: Microelectronic Devices and Circuits	Spring 2008

# MIDTERM EXAMINATION #2

Time allotted:  80 minutes

NAME:  _________________________	_________________________	_________________________

(print)		Last				First				Signature

STUDENT ID#:	____________________

INSTRUCTIONS:

- Use the values of physical constants provided below.

- SHOW YOUR WORK.  (Make your methods clear to the grader!)

- Clearly mark (underline or box) your answers.

- Specify the units on answers whenever appropriate.

| PHYSICAL CONSTANTS. Description; Symbol; Value. Electronic charge; q; 1.610-19 C. Boltzmann’s constant; k; 8.6210-5 eV/K. Thermal voltage at 300K; VT = kT/q; 0.026 V. Note that VT ln(10) = 0.060 V at T=300K. | PROPERTIES OF SILICON AT 300K. Description; Symbol; Value. Band gap energy; EG; 1.12 eV. Intrinsic carrier concentration; ni; 1010 cm-3. Dielectric permittivity; Si; 1.010-12 F/cm. |
| --- | --- |

Electron and Hole Mobilities in Silicon at 300K

SCORE:

Problem 1 [20 points]: MOS Amplifiers

- For this problem, use the following parameters for all NMOS transistors: 
VTH = 0.4 V, μnCox = 200 μA/V2, λ = 0.1V-1, (W/L)1 = (W/L)2 = 10. The current source is ideal.

  - Find the small signal parameters for M1 and M2 (gm, r0).  [4pts]

  - What is the topology of “Amplifier-A” (i.e., common source, common base, etc)? [2pts]

  - Find the voltage gain, input and output resistance of Amplifier-A. Show both the expressions and the numerical values. You can make approximations in your expression as long as they are within 10% accuracy. [6pts]

  - What is the topology of “Amplifier-B” (i.e., common source, common base, etc)? [2pts]

  - Find the voltage gain, input and output resistance of Amplifier-B. Show both the expressions and the numerical values. You can make approximations in your expression as long as they are within 10% accuracy. [6pts]

Problem 2 [25 points]: BJT Circuits and Frequency Response

- Assume that VCC = 3 V, IREF = 100µA, IS = 10−17 A, VA = 50 V, and β = 100 for all transistors. AE1 = AE2 = 10AE3. RC = 1 kΩ, and RE = 1 kΩ. Assume Cμ = 10 fF, Cπ = 100 fF, and CCS = 20 fF.

  - Identify the functions of Q1, Q2, and Q3. What is the function of this circuit? [3pts]

  - Find the small-signal parameters of the main amplifier transistor (). [3pts]

  - Find the expression and the value of the voltage gain. [4pts]

  - Show all parasitic capacitances of the BJTs in the circuit diagram above. Simplify the capacitances (e.g., combine all capacitances in parallel, remove capacitances that are shorted). Redraw the circuit diagram below with the simplified capacitances.  [Hint: a constant DC voltage is AC ground].  [5pts]

  - Find the input and output poles of this circuits. What is the dominant pole? Find the 3-dB bandwidth of this circuit. [5pts].

  - Construct the Bode plot of the transistor. Clearly mark the scale of both axes. The Bode plot should show both the low-frequency voltage gain as well as 3-dB bandwidth of the amplifier. [5pts]

Problem 3 [15 points]: MOS Devices

- Below is the cross section of a PMOS transistor:

  - What is the doping type (n, n+, p, or p+, where “+” means high doping concentration) of [3pts]

    - Source:

    - Drain:

    - Substrate (body):

  - Which carrier(s) are involved in current conduction? (i) electrons, (ii) holes), (iii) both electrons and holes. (choose one) [3pts]

  - If the power supply voltage is 2V and ground is 0V, what bias voltage is usually connected to the body (substrate) of the transistor? Why? [3pts]

  - Assume the threshold voltage of the PMOS is VTH = -0.4V. If VS = 2V, VD = 0V, find the gate voltages for which the PMOS is (i) cut-off, (ii) in between saturation and triode regions. [3pts]

  - If a PMOS and an NMOS have exactly the same dimension (W, L, and oxide thickness), which transistor has higher ? Why? (Assume both are long-channel devices) [3pts]

Problem 4 [20 points]: Cascode Amplifiers

- Below is a cascode amplifier with mixed MOS and BJT transistors:

MOS transistor parameters: VTH = 0.5V, μnCox = 200 μA/V2, λ = 0.1V-1, (W/L)1 = (W/L)2 = 10

BJT transistor parameters:  IS = 10−17 A, VA = 10 V, and β = 100

  - Find the small-signal parameters of the MOS transistor (). [2pts]

  - Find the small-signal parameters of the BJT transistor (). [3pts]

  - From a) and b), find the relative magnitudes of small-signal parameters: [3pts]

  - Derive the expressions of the output resistances for both cascade amplifiers shown above. You can drop small terms that are less than 10% of the dominant terms. Using the ratios you obtained in c), determine which cascode amplifier (Cascode-A or Cascode-B) has higher output resistance. [6pts]

  - Derive the expressions of the voltage gain for both cascade amplifiers shown above. You can drop small terms that are less than 10% of the dominant terms. Using the ratios you obtained in c), determine which cascode amplifier (Cascode-A or Cascode-B) has higher voltage gain. [6pts]